TECHNICAL REPORT

Vision-Based Cup Volume Estimation

Dual-Nozzle Block  |  Wide-Angle Fisheye Stereo  |  Sub-Pixel Signal A/B

Dynamic Resolution  |  Stereo-Bootstrapped Auto-Calibration Wizard

| System | Dual-nozzle coffee machine (Jura X10-style block) |
| --- | --- |
| Camera Count | 2 — stereo pair, fixed 3 cm baseline on nozzle block |
| Lens Type | Wide-angle / fisheye 128–160 deg FoV |
| Resolution | Dynamic — 80x60 / 640x480 / 1280x720 (configurable) |
| Nozzle Drive | Manual (user knob) — no motor/encoder |
| Cup Configuration | 1 or 2 cups — auto-detected |
| Calibration Method | Stereo-bootstrapped wizard — no ruler needed |
| Report Version | 2.0 (adds: dynamic resolution, auto-cal wizard) |
| Date | Wed Mar 25 2026 |

# 1. System Overview

This report documents the complete algorithm pipeline for vision-based cup volume estimation on a dual-nozzle coffee machine. Version 2 adds three major enhancements over v1: (1) dynamic resolution support so the same algorithm runs unchanged at 80x60, VGA, or HD; (2) normalized Signal B that is resolution-independent; and (3) a fully automated stereo-bootstrapped calibration wizard that requires no physical ruler.

## 1.1 What Changes Between Resolutions

When camera resolution changes, the underlying algorithm logic does not change. Only the following values change:

| Parameter | 80x60 | 640x480 VGA | 1280x720 HD | How to Handle |
| --- | --- | --- | --- | --- |
| Row count (H) | 60 | 480 | 720 | Scale ROI rows by H/60 |
| Col count (W) | 80 | 640 | 1280 | Scale split col by W/2 |
| Focal length f (px) | ~70 | ~560 | ~1120 | Re-measure after stereo cal |
| Signal A slope m | e.g. 0.747 | Different value | Different value | Auto-cal wizard |
| Signal A intercept c | e.g. -8.28 | Different value | Different value | Auto-cal wizard |
| Signal B (dark count) | 0-3200 px | 0-614400 px | 0-2764800 px | Normalize to ratio |
| Signal B m_b / c_b | Resolution-specific | Resolution-specific | Resolution-specific | Auto-cal wizard |

Key Insight The 0.60 adaptive threshold ratio, the sub-pixel interpolation formula, the stereo disparity formula, and the volume formula are all resolution-independent. Only spatial ROI indices and calibration constants (m, c, m_b, c_b, f) need to be re-derived per resolution.

# 2. Dynamic Resolution — CameraConfig Class

All spatial parameters are encapsulated in a single CameraConfig object. Instantiate it once with the camera resolution and all algorithm functions consume it. No hardcoded pixel values exist anywhere in the pipeline.

## 2.1 CameraConfig Implementation

class CameraConfig: """ Resolution-aware configuration for the vision pipeline. All spatial parameters scale automatically from the 80x60 baseline. Calibration constants (m, c, f, etc.) are NOT derived here — they are measured by the calibration wizard (Section 4). """ # Reference resolution used during original algorithm design BASE_W, BASE_H = 80, 60 def __init__(self, width: int, height: int): self.W = width self.H = height sx = width / self.BASE_W # horizontal scale factor sy = height / self.BASE_H # vertical scale factor # ── Signal A/B spatial zones (scale from 80x60 baseline) ── self.BRIGHT_START = max(1, round(5 * sy)) # bright zone top row self.BRIGHT_END = max(2, round(20 * sy)) # bright zone bottom row self.SEARCH_START = round(20 * sy) # transition search start self.SEARCH_END = height - 1 # transition search end self.SPLIT_COL = width // 2 # dual-cup split column # ── Stereo ──────────────────────────────────────────────── self.BASELINE_CM = 3.0 # physical, fixed, not resolution-dependent # ── Plausibility bounds (physical, not pixel-based) ─────── self.H_NOZ_RANGE = (5.5, 12.5) # cm self.Z_RIM_RANGE = (0.5, 13.0) # cm self.H_CUP_RANGE = (3.0, 15.0) # cm self.W_CUP_RANGE = (4.0, 13.0) # cm self.VOL_RANGE = (40, 800 ) # mL def half_w(self): return self.W // 2 def half_h(self): return self.H // 2 def __repr__(self): return (f'CameraConfig({self.W}x{self.H}) ' f'bright=[{self.BRIGHT_START}:{self.BRIGHT_END}] ' f'search=[{self.SEARCH_START}:{self.SEARCH_END}] ' f'split_col={self.SPLIT_COL})') # ── Preset instances ───────────────────────────────────────────── CAM_80x60 = CameraConfig(80, 60 ) # original CAM_VGA = CameraConfig(640, 480) # recommended CAM_HD = CameraConfig(1280, 720) # high accuracy

## 2.2 Accuracy Improvement by Resolution

| Resolution | Row Mean Width | H_nozzle Noise Reduction | Stereo Error at Z=10cm | Recommended Use |
| --- | --- | --- | --- | --- |
| 80x60 | 80 px (40 split) | Baseline | +/- 0.48 cm / px | Compute-constrained edge |
| 640x480 | 640 px (320 split) | 2.8x lower | +/- 0.06 cm / px | Recommended sweet spot |
| 1280x720 | 1280 px (640 split) | 4.0x lower | +/- 0.03 cm / px | Highest accuracy |

# 3. Stereo Fisheye Calibration — Chessboard Tutorial

This is the foundational one-time calibration. It must be completed before the Signal A/B calibration wizard (Section 4) can run, because the wizard uses stereo depth as its ground truth. This procedure produces the rectification maps that correct fisheye distortion and align both camera feeds to the same horizontal scanline.

## 3.1 Equipment Required

- Printed chessboard pattern: 9x7 inner corners, each square exactly 25 mm. Print on matte paper, mount on rigid flat board. Verify printed square size with a ruler before use.

- The physical nozzle block hardware with both cameras installed and connected.

- A laptop or PC running the calibration script (does not need to be the edge device).

- Good, diffuse lighting — avoid direct sunlight or shadows across the chessboard.

## 3.2 Step-by-Step Chessboard Capture

### Step 1 — Prepare the capture script:

import cv2, numpy as np, glob, pickle CHESSBOARD = (9, 7) # inner corners (cols, rows) SQUARE_MM = 25.0 # physical square size in mm N_TARGET = 30 # aim for 30+ good stereo pairs IMG_SIZE = (640, 480) # match your actual camera resolution # 3D object points for one chessboard view objp = np.zeros((CHESSBOARD[0]*CHESSBOARD[1], 3), np.float32) objp[:, :2] = np.mgrid[0:CHESSBOARD[0], 0:CHESSBOARD[1]].T.reshape(-1, 2) objp *= SQUARE_MM obj_pts, img_pts_L, img_pts_R = [], [], [] criteria = (cv2.TERM_CRITERIA_EPS + cv2.TERM_CRITERIA_MAX_ITER, 30, 0.001)

### Step 2 — Capture synchronized stereo pairs:

Hold the chessboard flat (or slightly angled) within the nozzle working volume — between 6 cm and 14 cm below the cameras. Move through as many positions and angles as possible. The system will only save pairs where corners are found in BOTH frames.

cap_L = cv2.VideoCapture(0) # Camera Left index cap_R = cv2.VideoCapture(1) # Camera Right index saved = 0 print(f'Move chessboard freely. Need {N_TARGET} good pairs. Press Q to finish.') while saved < N_TARGET: ret_L, frame_L = cap_L.read() ret_R, frame_R = cap_R.read() if not ret_L or not ret_R: continue gray_L = cv2.cvtColor(frame_L, cv2.COLOR_BGR2GRAY) gray_R = cv2.cvtColor(frame_R, cv2.COLOR_BGR2GRAY) found_L, corners_L = cv2.findChessboardCorners(gray_L, CHESSBOARD, None) found_R, corners_R = cv2.findChessboardCorners(gray_R, CHESSBOARD, None) if found_L and found_R: # Refine to sub-pixel accuracy corners_L = cv2.cornerSubPix(gray_L, corners_L, (5,5), (-1,-1), criteria) corners_R = cv2.cornerSubPix(gray_R, corners_R, (5,5), (-1,-1), criteria) obj_pts.append(objp) img_pts_L.append(corners_L) img_pts_R.append(corners_R) saved += 1 print(f' Saved pair {saved}/{N_TARGET}') cv2.waitKey(500) # small pause to get different positions # Show preview vis = np.hstack([frame_L, frame_R]) if found_L and found_R: cv2.drawChessboardCorners(frame_L, CHESSBOARD, corners_L, found_L) cv2.drawChessboardCorners(frame_R, CHESSBOARD, corners_R, found_R) cv2.imshow('Stereo Calibration (L ¦ R)', vis) if cv2.waitKey(1) & 0xFF == ord('q'): break cap_L.release(); cap_R.release(); cv2.destroyAllWindows()

### Step 3 — Run fisheye stereo calibration:

Use the Kannala-Brandt fisheye model for lenses wider than 120 degrees. This model accurately handles equidistant projection, which standard radial models cannot.

flags_single = (cv2.fisheye.CALIB_RECOMPUTE_EXTRINSIC ¦ cv2.fisheye.CALIB_FIX_SKEW ¦ cv2.fisheye.CALIB_CHECK_COND) K_L=np.zeros((3,3)); D_L=np.zeros((4,1)) K_R=np.zeros((3,3)); D_R=np.zeros((4,1)) # Calibrate each camera individually first cv2.fisheye.calibrate(obj_pts, img_pts_L, IMG_SIZE, K_L, D_L, flags=flags_single) cv2.fisheye.calibrate(obj_pts, img_pts_R, IMG_SIZE, K_R, D_R, flags=flags_single) # Stereo calibration — get R (rotation) and T (translation = 3cm baseline) ret, K_L, D_L, K_R, D_R, R, T = cv2.fisheye.stereoCalibrate( obj_pts, img_pts_L, img_pts_R, K_L, D_L, K_R, D_R, IMG_SIZE, flags=cv2.fisheye.CALIB_FIX_INTRINSIC ) print(f'Stereo RMS reprojection error: {ret:.4f} px (target < 1.0)') # Compute rectification transforms R_L,R_R,P_L,P_R,Q = cv2.fisheye.stereoRectify( K_L,D_L,K_R,D_R,IMG_SIZE,R,T, flags=cv2.CALIB_ZERO_DISPARITY,balance=1.0 ) # Build pixel-to-pixel remap lookup tables (fast at runtime) map_Lx,map_Ly = cv2.fisheye.initUndistortRectifyMap( K_L,D_L,R_L,P_L,IMG_SIZE,cv2.CV_32FC1) map_Rx,map_Ry = cv2.fisheye.initUndistortRectifyMap( K_R,D_R,R_R,P_R,IMG_SIZE,cv2.CV_32FC1) # Save everything to disk stereo_cal = dict(K_L=K_L,D_L=D_L,K_R=K_R,D_R=D_R,R=R,T=T, R_L=R_L,R_R=R_R,P_L=P_L,P_R=P_R,Q=Q, map_Lx=map_Lx,map_Ly=map_Ly, map_Rx=map_Rx,map_Ry=map_Ry, img_size=IMG_SIZE) pickle.dump(stereo_cal, open('stereo_calibration.pkl','wb')) print('Stereo calibration saved.')

### Step 4 — Verify calibration quality:

A well-calibrated stereo pair will have corresponding points at the same vertical (Y) coordinate in both rectified frames. This is called the epipolar constraint.

# Verification: check epipolar constraint on a test pair test_L = cv2.remap(gray_L, map_Lx, map_Ly, cv2.INTER_LINEAR) test_R = cv2.remap(gray_R, map_Rx, map_Ry, cv2.INTER_LINEAR) # Draw horizontal epipolar lines — all matched features should lie ON these lines vis = cv2.cvtColor(np.hstack([test_L, test_R]), cv2.COLOR_GRAY2BGR) for y in range(0, IMG_SIZE[1], IMG_SIZE[1]//8): cv2.line(vis, (0,y), (IMG_SIZE[0]*2, y), (0,255,0), 1) cv2.imshow('Rectified pair — check that features align horizontally', vis) cv2.waitKey(0) # PASS criteria: # - Reprojection RMS error < 1.0 px # - Chessboard corners in rectified frames differ in Y by < 2 px # - No visible vertical misalignment between left and right frames

Recalibration Trigger If the nozzle block is dropped, disassembled, or either camera mount is disturbed, stereo calibration MUST be repeated. A quick validation check (RMS error re-measurement) can confirm whether recalibration is needed without full re-capture.

# 4. Stereo-Bootstrapped Auto-Calibration Wizard

This wizard derives Signal A/B constants (m, c, m_b_norm, c_b_norm) and the focal length (f) automatically — with no physical ruler and no hardcoded centimeter values. It uses stereo depth (Section 3) as ground truth for absolute distance, then observes Signal A/B responses to build the calibration regression.

## 4.1 Core Principle

PROBLEM: We need m and c for H_nozzle = m * R_trans + c but we don't know true H_nozzle without a ruler. SOLUTION: Stereo already gives us the distance from the camera to a flat reference target (chessboard): Z_target = (f_approx * B) / d_target And camera mount height H_cam is fixed and known from the machine mechanical spec (e.g. 25 cm). Therefore: H_nozzle_true = H_cam - Z_target WIZARD: Technician moves nozzle to N positions manually. At each position, system captures ONE frame pair. System computes: R_trans <- Signal A/B H_true <- H_cam - Z_stereo After N positions: linear regression -> m, c Validation: R^2 > 0.999 -> save. Else: retry.

One Prerequisite The stereo calibration (Section 3) must be complete and the rectification maps must be loaded before the wizard can run. The wizard also needs H_cam — the fixed physical distance from the camera lens to the tray surface — entered once as a machine constant (measured with a ruler once during assembly, never changes).

## 4.2 Wizard Calibration Target

Any flat, high-contrast target works — the chessboard from Section 3 is ideal since it is already available. Place it flat on the tray. The wizard uses the center of the detected chessboard (or any flat reference surface) to measure Z_target via stereo.

## 4.3 Full Wizard Implementation

import numpy as np, pickle, cv2 from scipy.stats import linregress def run_calibration_wizard( cam: 'CameraConfig', map_Lx, map_Ly, map_Rx, map_Ry, # from stereo calibration focal_approx_px: float, # rough focal length estimate H_cam_cm: float, # fixed camera height above tray (cm) yolo_model, # YOLO for chessboard/target detection cam_L, cam_R, # camera capture objects n_positions: int = 5, # number of nozzle positions to capture dark_threshold: int = 40, ) -> dict: """ Guided calibration wizard. Returns calibration dict: {m, c, m_b_norm, c_b_norm, f} """ R_trans_list = [] H_true_list = [] dark_ratio_list= [] print('=== CALIBRATION WIZARD STARTED ===') print(f' Camera: {cam.W}x{cam.H} ¦ H_cam: {H_cam_cm} cm') print(f' Please place the flat calibration target on the tray.') print(f' You will be prompted to move the nozzle {n_positions} times.\n') for i in range(n_positions): if i == 0: print(f' Position {i+1}/{n_positions}: Move nozzle to LOWEST position.') elif i == n_positions-1: print(f' Position {i+1}/{n_positions}: Move nozzle to HIGHEST position.') else: print(f' Position {i+1}/{n_positions}: Move nozzle to an intermediate position.') input(' Press ENTER when nozzle is steady...') # Capture synchronized frames raw_L = cam_L.capture_grayscale() raw_R = cam_R.capture_grayscale() # Rectify for stereo rect_L = cv2.remap(raw_L, map_Lx, map_Ly, cv2.INTER_LINEAR) rect_R = cv2.remap(raw_R, map_Rx, map_Ry, cv2.INTER_LINEAR) # ── Get Z_target from stereo ────────────────────────────── # Detect target center X in each rectified frame # (use YOLO, or fallback: use image centroid of bright region) cx_L = _detect_target_center_x(rect_L, yolo_model) cx_R = _detect_target_center_x(rect_R, yolo_model) if cx_L is None or cx_R is None: print(' [WARN] Target not detected — skipping this position.') continue d = cx_L - cx_R if d <= 0: print(' [WARN] Invalid disparity — ensure target is visible.') continue Z_target = (focal_approx_px * cam.BASELINE_CM) / d H_true = H_cam_cm - Z_target if not (cam.H_NOZ_RANGE[0] <= H_true <= cam.H_NOZ_RANGE[1]): print(f' [WARN] H_true={H_true:.2f} cm out of range — skipping.') continue # ── Get R_trans from Signal A/B (raw frame) ─────────────── row_means = np.mean(raw_L, axis=1) I_max = np.max(row_means[cam.BRIGHT_START:cam.BRIGHT_END]) threshold = I_max * 0.60 R_trans = None for r in range(cam.SEARCH_START, cam.SEARCH_END): if row_means[r] >= threshold > row_means[r+1]: frac = (row_means[r]-threshold)/(row_means[r]-row_means[r+1]) R_trans = r + frac break if R_trans is None: print(' [WARN] No transition row found — check tray visibility.') continue # ── Get normalized dark ratio (Signal B) ────────────────── roi = raw_L[cam.SEARCH_START:, :] dark_ratio= np.sum(roi < dark_threshold) / roi.size R_trans_list.append(R_trans) H_true_list.append(H_true) dark_ratio_list.append(dark_ratio) print(f' Captured: R_trans={R_trans:.3f} H_true={H_true:.3f} cm') # ── Linear regression ───────────────────────────────────────── if len(R_trans_list) < 3: raise RuntimeError('Not enough valid captures. Need at least 3.') slope_A, intercept_A, r_A, _, _ = linregress(R_trans_list, H_true_list) slope_B, intercept_B, r_B, _, _ = linregress(dark_ratio_list, H_true_list) print(f'\n Signal A: m={slope_A:.6f} c={intercept_A:.6f} R^2={r_A**2:.6f}') print(f' Signal B: m_norm={slope_B:.4f} c_norm={intercept_B:.4f} R^2={r_B**2:.6f}') if r_A**2 < 0.999: raise RuntimeError( f'Signal A R^2={r_A**2:.4f} < 0.999. ' f'Check tray visibility and re-run wizard with more positions.' ) cal = { 'm' : float(slope_A), 'c' : float(intercept_A), 'm_b_norm' : float(slope_B), 'c_b_norm' : float(intercept_B), 'f' : focal_approx_px, # updated separately in Section 4.4 'H_cam_cm' : H_cam_cm, 'cam_w' : cam.W, 'cam_h' : cam.H, } pickle.dump(cal, open('signal_ab_calibration.pkl','wb')) print(' Calibration saved to signal_ab_calibration.pkl') return cal

## 4.4 Focal Length Auto-Measurement

Focal length f is measured once after stereo calibration. It can also be read directly from the P_L projection matrix output of stereoRectify (the most accurate method):

# Method 1: Read directly from stereoRectify output (most accurate) # P_L is the 3x4 projection matrix for the rectified left camera f_from_stereo = P_L[0, 0] # top-left element = focal length in pixels print(f'Focal length from stereo calibration: {f_from_stereo:.2f} px') # Method 2: Physical measurement (cross-check) # Place disc of known diameter W_real_cm flat on tray # Lower nozzle, run stereo to get Z_target # Measure W_pixels from YOLO bounding box on rectified frame # f = (W_pixels * Z_target) / W_real_cm # Both methods should agree within 2-3 px. # Use P_L[0,0] as primary value and save to calibration dict.

## 4.5 When to Re-Run the Wizard

| Event | Stereo Cal (Sec 3) | Wizard (Sec 4) | Reason |
| --- | --- | --- | --- |
| First unit assembly | Required | Required | Nothing calibrated yet |
| Camera mount disturbed | Required | Required | Both mappings invalidated |
| Lens replaced | Required | Required | K, D, f all change |
| Resolution changed | Required | Required | Pixel scaling changes |
| H_cam changed (tray height) | Not needed | Required | H_true reference shifts |
| Normal daily use | Not needed | Not needed | System stable |
| Service check (6 months) | Validate RMS only | Validate R^2 only | Catch drift early |

# 5. Signal A/B — Nozzle Height (Dynamic Resolution)

The algorithm is identical to v1 but all hardcoded pixel values are replaced by CameraConfig parameters, and Signal B now uses a normalized dark ratio instead of a raw pixel count, making it resolution-independent.

## 5.1 Normalized Signal B — Why and How

In v1, Signal B counted raw dark pixels. This count scales with image area (quadratically with resolution), making m_b and c_b completely different between resolutions. By dividing by total ROI area, we get a ratio between 0.0 and 1.0 that is physically meaningful and nearly resolution-independent.

v1 (resolution-dependent): P_dark = SUM{ I(r,c) < threshold } H_B = m_b * P_dark + c_b <- m_b changes with W*H v2 (resolution-independent): roi_area = (H - SEARCH_START) * W_roi dark_ratio = SUM{ I(r,c) < threshold } / roi_area <- 0.0 to 1.0 H_B = m_b_norm * dark_ratio + c_b_norm <- same m_b_norm at any res

## 5.2 Full Implementation

import numpy as np def measure_nozzle_height( frame_raw: np.ndarray, cam: 'CameraConfig', m: float, c: float, m_b_norm: float, c_b_norm: float, dark_threshold: int = 40, dual_cup: bool = False, ) -> dict: """ frame_raw : HxW grayscale, RAW (not rectified) — any resolution cam : CameraConfig instance Returns : {'L': H_cm or None, 'R': H_cm or None} """ assert frame_raw.shape == (cam.H, cam.W), ( f'Frame shape {frame_raw.shape} does not match CameraConfig {cam.W}x{cam.H}' ) def _compute_side(roi: np.ndarray): row_means = np.mean(roi, axis=1) # Adaptive threshold from bright zone bright_zone = row_means[cam.BRIGHT_START:cam.BRIGHT_END] if bright_zone.size == 0: return None I_max = np.max(bright_zone) threshold = I_max * 0.60 # Sub-pixel transition search R_trans = None for r in range(cam.SEARCH_START, cam.SEARCH_END): if row_means[r] >= threshold > row_means[r + 1]: denom = row_means[r] - row_means[r + 1] if abs(denom) < 1e-6: break frac = (row_means[r] - threshold) / denom R_trans = r + frac break if R_trans is None: return None # Signal A H_A = m * R_trans + c # Signal B — normalized dark ratio search_roi = roi[cam.SEARCH_START:, :] dark_ratio = np.sum(search_roi < dark_threshold) / search_roi.size H_B = m_b_norm * dark_ratio + c_b_norm return 0.70 * H_A + 0.30 * H_B if dual_cup: return { 'L': _compute_side(frame_raw[:, :cam.SPLIT_COL]), 'R': _compute_side(frame_raw[:, cam.SPLIT_COL:]), } return {'L': _compute_side(frame_raw), 'R': None}

# 6. Stereo Depth — Rim Distance Z_rim

The stereo depth formula is fully resolution-independent. Only f (focal length in pixels) changes with resolution — and it is read from the stereo calibration output (P_L[0,0]) rather than hardcoded.

Z_rim = (f * B) / d f = P_L[0,0] from stereoRectify (auto, resolution-aware) B = 3.0 cm (fixed physical baseline) d = X_left - X_right (YOLO centers on rectified frames)

## 6.1 Implementation

def measure_z_rim_dual( boxes_L: list, boxes_R: list, cam: 'CameraConfig', focal_px: float, ) -> dict: boxes_L = sorted(boxes_L, key=lambda b: b['cx']) boxes_R = sorted(boxes_R, key=lambda b: b['cx']) def _z(bL, bR): d = bL['cx'] - bR['cx'] if d <= 1.0: return None z = (focal_px * cam.BASELINE_CM) / d lo, hi = cam.Z_RIM_RANGE return z if lo <= z <= hi else None result = {'cup1': None, 'cup2': None} if len(boxes_L) >= 1 and len(boxes_R) >= 1: result['cup1'] = _z(boxes_L[0], boxes_R[0]) if len(boxes_L) >= 2 and len(boxes_R) >= 2: result['cup2'] = _z(boxes_L[1], boxes_R[1]) return result

# 7. Full Integration Pipeline

import cv2, numpy as np, math, pickle # ── Load configs ──────────────────────────────────────────────── cam = CameraConfig(640, 480) # or 80x60, 1280x720 st = pickle.load(open('stereo_calibration.pkl','rb')) cal = pickle.load(open('signal_ab_calibration.pkl','rb')) map_Lx = st['map_Lx']; map_Ly = st['map_Ly'] map_Rx = st['map_Rx']; map_Ry = st['map_Ry'] focal = st['P_L'][0,0] # resolution-aware focal length def run_pipeline(cam_L, cam_R, yolo_model): # 1. Capture raw_L = cam_L.capture_grayscale() # HxW, raw raw_R = cam_R.capture_grayscale() # 2. Rectify rect_L = cv2.remap(raw_L, map_Lx, map_Ly, cv2.INTER_LINEAR) rect_R = cv2.remap(raw_R, map_Rx, map_Ry, cv2.INTER_LINEAR) # 3. YOLO on rectified frames boxes_L = yolo_model.detect(rect_L, cls='cup_rim') boxes_R = yolo_model.detect(rect_R, cls='cup_rim') # 4. Cup configuration cfg = detect_cup_configuration(boxes_L, cam) if cfg['mode'] == 'NO_CUP': return None, 'No cup detected' dual = (cfg['mode'] == 'DUAL_CUP') # 5. H_nozzle — Signal A/B on RAW frame h_noz = measure_nozzle_height( raw_L, cam, m=cal['m'], c=cal['c'], m_b_norm=cal['m_b_norm'], c_b_norm=cal['c_b_norm'], dual_cup=dual ) # 6. Z_rim — stereo on RECTIFIED frames z_rim = measure_z_rim_dual(boxes_L, boxes_R, cam, focal) results = {} for i, side in enumerate(['cup1'] if not dual else ['cup1','cup2']): sk = 'L' if i == 0 else 'R' H_n = h_noz.get(sk); Z_r = z_rim.get(side) if H_n is None or Z_r is None: results[side] = {'error': 'Measurement failed'}; continue # 7. Cup height H_cup = H_n - Z_r # 8. Diameter box = sorted(boxes_L, key=lambda b:b['cx'])[i] W_real = (box['w'] * Z_r) / focal # 9. Volume volume = math.pi * (W_real/2.0)**2 * H_cup # 10. Plausibility meas = {'H_nozzle_cm':H_n,'Z_rim_cm':Z_r,'H_cup_cm':H_cup, 'W_real_cm':W_real,'volume_mL':volume} ok = all( lo <= meas[k] <= hi for k,(lo,hi) in [ ('H_nozzle_cm',cam.H_NOZ_RANGE),('Z_rim_cm',cam.Z_RIM_RANGE), ('H_cup_cm',cam.H_CUP_RANGE),('W_real_cm',cam.W_CUP_RANGE), ('volume_mL',cam.VOL_RANGE), ] ) results[side] = {k:round(v,2) for k,v in meas.items()} if ok \ else {'error':'Plausibility failed'} return results, 'OK'

# 8. Error Budget by Resolution

| Measurement | 80x60 | 640x480 VGA | 1280x720 HD |
| --- | --- | --- | --- |
| H_nozzle (full-width) | +/- 0.12 cm | +/- 0.04 cm | +/- 0.03 cm |
| H_nozzle (split ROI) | +/- 0.20 cm | +/- 0.07 cm | +/- 0.05 cm |
| Z_rim at Z=5 cm | +/- 0.12 cm | +/- 0.015 cm | +/- 0.008 cm |
| Z_rim at Z=10 cm | +/- 0.48 cm | +/- 0.060 cm | +/- 0.030 cm |
| H_cup (total) | +/- 0.60 cm | +/- 0.10 cm | +/- 0.06 cm |
| Volume (250 mL cup) | +/- 40 mL | +/- 8 mL | +/- 5 mL |

VGA (640x480) is strongly recommended. It delivers 5x better volume accuracy than 80x60 with a compute cost that remains feasible on modern edge hardware, and Signal A/B noise drops 2.8x due to wider row means.

# 9. Calibration Quick Reference Card

Summary of all calibration procedures, outputs, and conditions for when each must be re-run.

| Step | Procedure | Output | Tool/Method | Re-run When |
| --- | --- | --- | --- | --- |
| 1 | Fisheye stereo calibration | K_L,D_L,K_R,D_R,R,T,rect maps | Chessboard 9x7, 25mm squares, 30+ pairs | Camera moved or replaced |
| 2 | Focal length extraction | f (pixels) | P_L[0,0] from stereoRectify | After step 1 |
| 3 | Signal A/B wizard | m, c, m_b_norm, c_b_norm | Guided wizard — 5 nozzle positions | Resolution change, H_cam change, after step 1 |

# 10. Summary

This v2 pipeline is fully resolution-independent, automatically calibrated, and handles 0/1/2 cups with a single 2-camera setup on a rigid dual-nozzle block.

- CameraConfig class encapsulates all spatial parameters. Changing resolution requires only instantiating a different CameraConfig — no code changes.

- Signal B now uses a normalized dark ratio (0.0-1.0) instead of raw pixel count. The constants m_b_norm and c_b_norm are valid at any resolution, verified by the calibration wizard.

- The auto-calibration wizard requires no ruler. It uses stereo depth as ground truth and linear regression to derive m and c automatically in under 5 minutes.

- Stereo fisheye calibration using the Kannala-Brandt model correctly handles 128-160 degree lenses that standard radial models cannot.

- Focal length f is read directly from P_L[0,0] output of stereoRectify, eliminating manual measurement.

- Recommended production resolution: 640x480 VGA — delivers volume error of +/- 8 mL on a 250 mL cup while remaining feasible on DRP-AI edge hardware.

Calibration Summary for Field Technician (1) Run chessboard stereo capture script with 30+ pairs. (2) Run stereo calibration script — check RMS < 1.0. (3) Place chessboard flat on tray. (4) Launch calibration wizard — move nozzle to 5 positions when prompted. (5) Check wizard output: R^2 > 0.999. Done. Total time: approximately 10 minutes.