TECHNICAL REPORT

Vision-Based Cup Volume Estimation

Dual-Nozzle Block  |  Wide-Angle Stereo Cameras  |  Sub-Pixel Signal A/B Fusion

for Edge Deployment on Renesas DRP-AI

| System Target | Dual-nozzle coffee machine (Jura X10-style block) |
| --- | --- |
| Camera Count | 2 (stereo pair, fixed 3 cm baseline on nozzle block) |
| Lens Type | Wide-angle / fisheye 128°–160° FoV |
| Resolution | 80 x 60 pixels per camera |
| Nozzle Outputs | 2 (left + right), same block, move as one unit |
| Cup Configuration | 1 or 2 cups (system auto-detects) |
| Cup Sizes | Paired (always same size when 2 cups) |
| Target Height Error | < ±1.1 cm |
| Target Volume Error | < ±40 mL |
| Processing Platform | Edge device — Renesas DRP-AI + host CPU |
| Date | Wed Mar 25 2026 |

# 1. System Overview

This report documents a complete algorithm pipeline for estimating the physical volume of one or two cups placed under a dual-nozzle coffee machine. The nozzle block (containing both nozzles and both cameras) moves as a single rigid unit — up and down — driven by the user. All computation is optimized for edge deployment.

## 1.1 Physical Hardware Layout

TOP VIEW (looking down at tray): ┌────────────────────────────────────────────┐ │ TRAY SURFACE │ │ TRAY ┌────────┐ ┌────────┐ TRAY │ │ (visible)│ CUP L │ │ CUP R │(visible) │ │ │ rim │ │ rim │ │ │ └────────┘ └────────┘ │ │ TRAY (visible below cups) │ └────────────────────────────────────────────┘ SIDE VIEW (nozzle block moves up/down as one unit): ┌──────────────────────────────────────┐ │ NOZZLE BLOCK │ ← moves up/down │ [CAM-L]←──3cm──→[CAM-R] │ ← stereo pair │ [NZ-L ] [NZ-R ] │ ← 2 nozzle outputs └──────────────────────────────────────┘ ↓ wide-angle view ↓ [CUP L] [CUP R] ← 1 or 2 cups ══════════════ TRAY ══════════════

## 1.2 Core Measurement Strategy

Two independent measurements are made from the same reference point (the nozzle block / camera position), then combined:

- H_nozzle: Distance from tray surface UP to the camera/nozzle block. Measured by Signal A/B sub-pixel algorithm on CAM-L using the visible tray area as a contrast reference. Accuracy: +/-0.12 cm.

- Z_rim: Distance from the camera/nozzle block DOWN to the cup rim. Measured by stereo disparity using both CAM-L and CAM-R with YOLO rim detection. Accuracy: +/-0.5 to 1.0 cm.

- H_cup = H_nozzle - Z_rim. Independent of nozzle height — the lazy user problem is fully eliminated.

Key Design Decision CAM-L serves dual duty: (1) it is the primary camera for Signal A/B tray-based height measurement, and (2) it is the left eye of the stereo pair for disparity. No third camera is needed.

# 2. Wide-Angle / Fisheye Lens Considerations

Using a 128-160 degree wide-angle or fisheye lens is required because the nozzle block is physically close to the cups, and both cups must fit within a single camera frame. However, this introduces distortion that must be handled carefully per algorithm component.

## 2.1 Impact Per Algorithm Component

| Component | Affected by Fisheye? | Action Required |
| --- | --- | --- |
| Signal A/B row means | NO — row means are spatial averages | No change needed. Calibration implicitly absorbs vertical distortion. |
| Stereo rectification | YES — critical | Must use fisheye stereo calibration model (Kannala-Brandt). |
| YOLO rim detection | MINOR — bbox slightly warped | Apply undistortion before YOLO inference. |
| Pinhole diameter formula | YES — invalid without correction | Only valid after undistortion. Use rectified frames. |
| Z_rim disparity calc | YES | Must operate on rectified frames only. |

## 2.2 Why Signal A/B Is Safe with Wide Lens

Signal A/B computes the mean pixel intensity across all 80 columns for each row. Wide-angle distortion bends horizontal lines, but it does not change the statistical mean brightness of a row. The bright tray zone remains bright and the dark nozzle/cup zone remains dark. The transition point shifts predictably with nozzle height regardless of lens distortion, and the linear calibration fit (H = m * R_trans + c) absorbs any vertical distortion implicitly.

Important Constraint Signal A/B calibration must be performed with the exact final lens installed. If the lens is swapped, full recalibration of m and c is required.

## 2.3 Fisheye Calibration Model

For lenses wider than 100 degrees, the standard radial distortion model (Brown-Conrady) is insufficient. Use the Kannala-Brandt equidistant fisheye model available in OpenCV's fisheye module.

# Fisheye stereo calibration (run once at factory) import cv2 import numpy as np # Calibrate each camera individually first # obj_pts: 3D chessboard corners, img_pts_L/R: detected 2D corners flags = cv2.fisheye.CALIB_RECOMPUTE_EXTRINSIC ¦ cv2.fisheye.CALIB_FIX_SKEW ret, K_L, D_L, _, _ = cv2.fisheye.calibrate( obj_pts, img_pts_L, (80, 60), None, None, flags=flags ) ret, K_R, D_R, _, _ = cv2.fisheye.calibrate( obj_pts, img_pts_R, (80, 60), None, None, flags=flags ) # Stereo calibration to get R (rotation) and T (translation = 3cm baseline) ret, K_L, D_L, K_R, D_R, R, T = cv2.fisheye.stereoCalibrate( obj_pts, img_pts_L, img_pts_R, K_L, D_L, K_R, D_R, (80, 60), flags=cv2.fisheye.CALIB_FIX_INTRINSIC ) # Compute rectification maps (saved as lookup tables) R_L, R_R, P_L, P_R, Q = cv2.fisheye.stereoRectify( K_L, D_L, K_R, D_R, (80, 60), R, T, flags=cv2.CALIB_ZERO_DISPARITY, balance=1.0 ) map_Lx, map_Ly = cv2.fisheye.initUndistortRectifyMap( K_L, D_L, R_L, P_L, (80, 60), cv2.CV_32FC1 ) map_Rx, map_Ry = cv2.fisheye.initUndistortRectifyMap( K_R, D_R, R_R, P_R, (80, 60), cv2.CV_32FC1 ) # Save: K_L, D_L, K_R, D_R, R, T, map_Lx, map_Ly, map_Rx, map_Ry, Q

Resolution Loss After Undistortion At 80x60 with a 160-degree fisheye lens, undistortion/rectification will crop the corners of the frame. Effective usable resolution may reduce to approximately 55x45 pixels in the center region. Signal A/B is not affected (it uses the raw frame). Stereo and YOLO must operate on the rectified frame.

# 3. Calibration Procedure (One-Time Factory Setup)

All calibration is performed once per hardware unit with the exact final lens installed. Values are saved to a system configuration file. Recalibration is only needed if the camera mount is physically disturbed or the lens is replaced.

## 3.1 Signal A/B Linear Calibration

Establishes the pixel-to-centimeter mapping for nozzle height using the tray as a fixed reference.

### Procedure:

- Use a physical ruler to measure nozzle block height above the tray surface.

- Move the nozzle block to at least 4 known heights spanning the full travel range (e.g., 6.5, 8.0, 9.5, 11.5 cm).

- At each height, capture a raw (non-undistorted) grayscale frame from CAM-L.

- Run the Signal A/B algorithm (Section 4) to extract R_trans for each frame.

- Plot (R_trans, real_height) and fit a linear regression line.

H_nozzle = m * R_trans + c Solve: m = (n*SUM(xy) - SUM(x)*SUM(y)) / (n*SUM(x^2) - SUM(x)^2) c = (SUM(y) - m*SUM(x)) / n Target: R-squared > 0.999

### Sample Calibration Data:

| Real Height (cm) | R_trans (px) | Fitted H (cm) | Error (cm) |
| --- | --- | --- | --- |
| 6.5 | 19.716 | 6.49 | -0.01 |
| 8.0 | 21.724 | 7.98 | -0.02 |
| 9.5 | 23.731 | 9.49 | -0.01 |
| 11.5 | 26.390 | 11.47 | -0.03 |

## 3.2 Fisheye Stereo Calibration

Refer to Section 2.3 for the full code. In summary:

- Print a chessboard pattern (7x6 inner corners, 25 mm squares).

- Capture 25+ synchronized stereo image pairs at different angles within the cup working volume.

- Run cv2.fisheye.stereoCalibrate() to obtain K_L, D_L, K_R, D_R, R, T.

- Compute and save rectification maps (map_Lx, map_Ly, map_Rx, map_Ry).

- Verify: in a rectified stereo pair, a cup rim point in CAM-L and CAM-R must share the same Y-coordinate (epipolar constraint).

## 3.3 Focal Length for Diameter Estimation

- Place a circular disc of known diameter W_real (e.g., 8.0 cm) flat on the tray.

- Lower nozzle to a known height H_nozzle. The Z distance to the disc equals H_nozzle.

- Capture and rectify a CAM-L frame. Measure W_pixels (pixel width of disc).

f = (W_pixels * Z) / W_real

- Save f as a system constant (typically 60-80 px for wide-angle lens at 80px width).

# 4. Signal A/B — Nozzle Height Measurement (H_nozzle)

Runs on the raw (non-rectified) CAM-L frame. Uses the visible tray area as a high-contrast reference to locate the bright-to-dark transition row with sub-pixel precision.

## 4.1 Dual-Cup Frame Splitting

When two cups are present, they occupy the left and right halves of the frame. A naive full-width row mean would average across both cups and the gap between them, corrupting the transition signal. The frame is therefore split vertically at the midpoint column.

RAW CAM-L FRAME (80x60): col: 0 40 80 ┌─────────────┬─────────────┐ │ TRAY (bright) │ rows 5-20 │ ┌──────┐ │ ┌──────┐ │ │ │CUP L │ │ │CUP R │ │ rows 20-59 │ │(dark)│ │ │(dark)│ │ │ └──────┘ │ └──────┘ │ └─────────────┴─────────────┘ LEFT ROI RIGHT ROI cols 0-39 cols 40-79 Signal A/B L Signal A/B R

## 4.2 Mathematics

### Split Row Mean (width W_half = 40 px):

mu_r_L = (1/40) * SUM{ I(r,c) } for c = 0 to 39 mu_r_R = (1/40) * SUM{ I(r,c) } for c = 40 to 79

### Adaptive Threshold (per side):

I_max_L = max( mu_r_L ) for r in [5, 20] T_L = I_max_L * 0.60 I_max_R = max( mu_r_R ) for r in [5, 20] T_R = I_max_R * 0.60

### Sub-Pixel Transition (per side):

Find r where: mu_r >= T AND mu_{r+1} < T frac = (mu_r - T) / (mu_r - mu_{r+1}) R_trans = r + frac

### Height Conversion (shared calibration m, c):

H_nozzle = m * R_trans + c Note: Same m, c applies to both left and right ROI because nozzle block moves as one unit.

### Signal B (Dark Pixel Area per ROI):

P_dark_L = SUM{ I(r,c) < dark_threshold } for ROI-L rows 20-60 P_dark_R = SUM{ I(r,c) < dark_threshold } for ROI-R rows 20-60 H_B_L = m_b * P_dark_L + c_b H_B_R = m_b * P_dark_R + c_b

### Ensemble per side:

H_nozzle_L = 0.70 * H_A_L + 0.30 * H_B_L H_nozzle_R = 0.70 * H_A_R + 0.30 * H_B_R For single-cup mode: use only the active side.

## 4.3 Accuracy Impact of Frame Splitting

| Mode | Row Mean Width | Signal Noise | Expected H_nozzle Error |
| --- | --- | --- | --- |
| Single cup, full frame | 80 px | Low | +/- 0.12 cm |
| Dual cup, split ROI | 40 px | ~40% higher | +/- 0.20 cm |
| Single cup, split ROI (precaution) | 40 px | ~40% higher | +/- 0.20 cm |

Split ROI Tray Visibility Signal A/B requires visible bright tray in rows 5-20 of each ROI. If a very wide cup fully covers the top portion of one ROI, the I_max sampling zone will not capture the tray and the threshold will be wrong. Validate tray visibility per ROI before accepting the height reading.

## 4.4 Python Implementation

import numpy as np def measure_nozzle_height_split( frame_raw: np.ndarray, m: float, c: float, m_b: float, c_b: float, dark_threshold: int = 40, dual_cup: bool = False ) -> dict: """ frame_raw : 60x80 grayscale, RAW (not undistorted) dual_cup : True = split into L/R ROIs, False = full width Returns : {'L': H_cm or None, 'R': H_cm or None} """ def _compute_side(roi): row_means = np.mean(roi, axis=1) I_max = np.max(row_means[5:21]) threshold = I_max * 0.60 R_trans = None for r in range(20, roi.shape[0]-1): if row_means[r] >= threshold > row_means[r+1]: frac = (row_means[r] - threshold) / (row_means[r] - row_means[r+1]) R_trans = r + frac break if R_trans is None: return None H_A = m * R_trans + c P_dark = int(np.sum(roi[20:, :] < dark_threshold)) H_B = m_b * P_dark + c_b return 0.70 * H_A + 0.30 * H_B if dual_cup: return { 'L': _compute_side(frame_raw[:, :40]), 'R': _compute_side(frame_raw[:, 40:]), } else: h = _compute_side(frame_raw) return {'L': h, 'R': h}

# 5. Stereo Depth — Rim Distance (Z_rim)

Operates on rectified frames from both cameras. Uses YOLO rim detection on each frame to obtain the horizontal pixel position of the cup rim center, then computes depth via stereo disparity. This is computationally near-free since YOLO is already running.

## 5.1 Mathematics

Stereo depth formula: Z_rim = (f * B) / d where: f = focal length in pixels (from calibration, Section 3.3) B = stereo baseline = 3.0 cm (fixed, rigid nozzle block) d = disparity = X_left - X_right (pixels) X_left = YOLO rim center X in CAM-L rectified frame X_right = YOLO rim center X in CAM-R rectified frame

## 5.2 Dual-Cup Disparity Matching

When two cups are present, YOLO will detect two rim bounding boxes in each frame. Correct left-right matching is critical — the left cup in CAM-L must be matched to the left cup in CAM-R.

CAM-L rectified: [RIM-L at X=22] [RIM-R at X=58] CAM-R rectified: [RIM-L at X=16] [RIM-R at X=52] Sort boxes by X_center in each frame, then match by index: Cup 1 (left): d1 = 22 - 16 = 6 px -> Z_rim_1 = (f*3)/6 Cup 2 (right): d2 = 58 - 52 = 6 px -> Z_rim_2 = (f*3)/6 (Same Z because cups are the same height — paired cups.) Note: disparity may differ if cups are at slightly different lateral positions relative to camera center.

## 5.3 Expected Disparity vs. Accuracy

| Z_rim (cm) | Expected d (px) at f=70 | Error per 1px noise (cm) | Assessment |
| --- | --- | --- | --- |
| 4 | 52.5 | 0.08 | Excellent |
| 6 | 35.0 | 0.17 | Very good |
| 8 | 26.3 | 0.31 | Good |
| 10 | 21.0 | 0.48 | Acceptable |
| 12 | 17.5 | 0.69 | Borderline |
| 14 | 15.0 | 0.93 | Marginal |

## 5.4 Python Implementation

def measure_z_rim_dual( boxes_L: list, boxes_R: list, focal_px: float, baseline_cm: float = 3.0 ) -> dict: """ boxes_L, boxes_R : list of YOLO detections [{cx, cy, w, h, conf}, ...] on rectified frames, sorted by cx ascending Returns: {'cup1': Z_cm or None, 'cup2': Z_cm or None} """ def _z(box_l, box_r): d = box_l['cx'] - box_r['cx'] if d <= 1.0: return None # invalid disparity z = (focal_px * baseline_cm) / d return z if 2.0 <= z <= 15.0 else None # Sort both lists left-to-right by X center boxes_L = sorted(boxes_L, key=lambda b: b['cx']) boxes_R = sorted(boxes_R, key=lambda b: b['cx']) result = {'cup1': None, 'cup2': None} if len(boxes_L) >= 1 and len(boxes_R) >= 1: result['cup1'] = _z(boxes_L[0], boxes_R[0]) if len(boxes_L) >= 2 and len(boxes_R) >= 2: result['cup2'] = _z(boxes_L[1], boxes_R[1]) return result

# 6. Cup Count Detection & System State Machine

Before computing volume, the system must determine whether 0, 1, or 2 cups are present, and which side(s) are occupied. This drives the split-ROI logic for Signal A/B and the disparity matching for stereo.

## 6.1 Cup Count Logic

def detect_cup_configuration(boxes_L: list) -> dict: """ Uses YOLO detections from CAM-L (rectified) only. Returns cup configuration for this frame. """ boxes_L = sorted(boxes_L, key=lambda b: b['cx']) count = len(boxes_L) if count == 0: return {'mode': 'NO_CUP', 'left': False, 'right': False} FRAME_MID = 40 # midpoint of 80px frame if count == 1: cx = boxes_L[0]['cx'] if cx < FRAME_MID: return {'mode': 'SINGLE_LEFT', 'left': True, 'right': False} else: return {'mode': 'SINGLE_RIGHT', 'left': False, 'right': True} if count == 2: return {'mode': 'DUAL_CUP', 'left': True, 'right': True} return {'mode': 'UNKNOWN', 'left': False, 'right': False}

## 6.2 State Machine Overview

┌─────────────┐ │ IDLE/WAIT │ └──────┬──────┘ │ YOLO detects cup(s) ┌────────────┼────────────┐ ▼ ▼ ▼ ┌────────────┐ ┌─────────┐ ┌─────────────┐ │SINGLE LEFT │ │SINGLE │ │ DUAL CUP │ │ │ │RIGHT │ │ │ └─────┬──────┘ └────┬────┘ └──────┬──────┘ │ │ │ └──────────────┴─────────────┘ │ ┌────────▼────────┐ │ MEASURE PHASE │ │ Signal A/B │ │ Stereo Z_rim │ │ Volume calc │ └────────┬────────┘ │ ┌────────▼────────┐ │ PLAUSIBILITY │ │ CHECK │ └────────┬────────┘ PASS ▼ ▼ FAIL ┌──────────────┐ ┌───────────┐ │ DISPENSE CMD │ │ WARN USER │ └──────────────┘ └───────────┘

# 7. Cup Height, Diameter, and Volume Calculation

## 7.1 Cup Height

H_cup = H_nozzle - Z_rim Total error budget: H_nozzle error : +/- 0.12 cm (full-width) or +/- 0.20 cm (split ROI) Z_rim error : +/- 0.50 to 1.00 cm (stereo, 3cm baseline) ───────────────────────────────────────────────────────────── H_cup error : +/- 0.62 to 1.20 cm total (worst case)

## 7.2 Cup Diameter via Pinhole Projection

After rectification, the pinhole model is valid. Z_rim (from stereo) is used directly as the depth parameter, making the diameter estimate self-consistent with the height measurement.

W_real = (W_pixels * Z_rim) / f W_pixels = width of YOLO rim bounding box on CAM-L rectified frame Z_rim = stereo-measured distance to rim (cm) f = focal length (calibrated, px) W_real = physical cup diameter (cm)

## 7.3 Volume

R_cup = W_real / 2 V_cup = pi * R_cup^2 * H_cup (cylindrical approximation) = pi * (W_real / 2)^2 * H_cup Units: cm -> V in cm^3 = mL

## 7.4 Plausibility Gates

BOUNDS = { 'H_nozzle_cm' : (5.5, 12.5), 'Z_rim_cm' : (0.5, 13.0), 'H_cup_cm' : (3.0, 15.0), 'W_real_cm' : (4.0, 13.0), 'volume_mL' : (40, 800 ), } def plausibility_check(measurements: dict) -> bool: for key, (lo, hi) in BOUNDS.items(): val = measurements.get(key) if val is None or not (lo <= val <= hi): print(f'[FAIL] {key} = {val} (expected {lo}..{hi})') return False return True

# 8. Full Integration Pipeline

import cv2, numpy as np, math # ── Load calibration config ────────────────────────────────────── CFG = { 'm' : 0.7472, # Signal A slope 'c' : -8.2845, # Signal A intercept 'm_b' : -0.012943, # Signal B slope 'c_b' : 23.0693, # Signal B intercept 'f' : 70.0, # focal length (px) — post-rectification 'B' : 3.0, # stereo baseline (cm) } # Rectification maps loaded from stereo calibration file map_Lx = ... # loaded from file map_Ly = ... map_Rx = ... map_Ry = ... def run_pipeline(cam_L, cam_R, yolo_model): # ── Step 1: Capture synchronized frames ────────────────────── raw_L = cam_L.capture_grayscale() # 60x80, raw raw_R = cam_R.capture_grayscale() # 60x80, raw # ── Step 2: Rectify for stereo + YOLO ──────────────────────── rect_L = cv2.remap(raw_L, map_Lx, map_Ly, cv2.INTER_LINEAR) rect_R = cv2.remap(raw_R, map_Rx, map_Ry, cv2.INTER_LINEAR) # ── Step 3: YOLO on both rectified frames ───────────────────── boxes_L = yolo_model.detect(rect_L, cls='cup_rim') # list of {cx,cy,w,h} boxes_R = yolo_model.detect(rect_R, cls='cup_rim') # ── Step 4: Cup configuration ───────────────────────────────── cfg = detect_cup_configuration(boxes_L) # Section 6.1 if cfg['mode'] == 'NO_CUP': return None, 'No cup detected' dual = (cfg['mode'] == 'DUAL_CUP') # ── Step 5: Signal A/B on RAW frame ─────────────────────────── h_nozzle = measure_nozzle_height_split( # Section 4.4 raw_L, m=CFG['m'], c=CFG['c'], m_b=CFG['m_b'], c_b=CFG['c_b'], dual_cup=dual ) # ── Step 6: Stereo Z_rim ────────────────────────────────────── z_rim = measure_z_rim_dual( # Section 5.4 boxes_L, boxes_R, focal_px=CFG['f'], baseline_cm=CFG['B'] ) results = {} for side in (['cup1'] if not dual else ['cup1', 'cup2']): side_key = 'L' if side == 'cup1' else 'R' H_n = h_nozzle.get(side_key) Z_r = z_rim.get(side) if H_n is None or Z_r is None: results[side] = {'error': 'Measurement failed'} continue # ── Step 7: Cup height ──────────────────────────────────── H_cup = H_n - Z_r # ── Step 8: Diameter (from rectified frame) ─────────────── box = boxes_L[0 if side=='cup1' else 1] W_real = (box['w'] * Z_r) / CFG['f'] # ── Step 9: Volume ──────────────────────────────────────── volume = math.pi * (W_real / 2.0) ** 2 * H_cup # ── Step 10: Plausibility ───────────────────────────────── meas = { 'H_nozzle_cm': H_n, 'Z_rim_cm': Z_r, 'H_cup_cm': H_cup, 'W_real_cm': W_real, 'volume_mL': volume } if not plausibility_check(meas): results[side] = {'error': 'Plausibility check failed'} continue results[side] = {k: round(v, 2) for k, v in meas.items()} return results, 'OK'

# 9. Error Budget Summary

| Measurement | Method | Best Case Error | Worst Case Error | Limiting Factor |
| --- | --- | --- | --- | --- |
| H_nozzle | Signal A/B full-width | +/- 0.12 cm | +/- 0.12 cm | Row mean noise |
| H_nozzle | Signal A/B split ROI | +/- 0.18 cm | +/- 0.25 cm | Halved sample width |
| Z_rim | Stereo (Z=5 cm) | +/- 0.12 cm | +/- 0.20 cm | 1-2px YOLO error |
| Z_rim | Stereo (Z=12 cm) | +/- 0.60 cm | +/- 1.00 cm | 1-2px YOLO error |
| H_cup | Subtraction fusion | +/- 0.30 cm | +/- 1.20 cm | Dominated by Z_rim |
| W_real (diameter) | Pinhole + Z_rim | +/- 0.50 cm | +/- 1.50 cm | YOLO bbox precision |
| Volume | pi*R^2*H | +/- 15 mL | +/- 50 mL | Propagated errors |

Volume error of +/-50 mL on a 300 mL cappuccino is approximately 17% — acceptable for a smart dispense system. For espresso shots below 60 mL, rely on cup classification (YOLO class label) rather than geometric volume calculation.

# 10. Known Limitations & Mitigations

| Issue | Impact | Mitigation |
| --- | --- | --- |
| Transparent / glass cups | YOLO misses rim or detects inner base | Add 'glass_rim' class to YOLO training set. Plausibility gate catches outliers. |
| Cup off-center laterally | Lens distortion warps W_pixels for diameter | Center validation: warn if rim center is >30% off horizontal midline of ROI. |
| Tray spill / contamination | Signal B dark pixel count inflated | Trigger measurement at brew-start only; lock H_nozzle value immediately. |
| Single cup placed on wrong side | Signal A/B split ROI reads wrong half | Cup config detection (Section 6.1) assigns side correctly via X-position. |
| Very wide mug (>13 cm) | Rim extends outside one ROI boundary | Detect via YOLO bbox width > 35px; apply preset volume for 'large mug' class. |
| Camera knocked out of alignment | Stereo rectification invalid | Include service-mode recalibration routine; monitor stereo reprojection error. |
| Effective resolution loss (fisheye) | Rectification crops corners; ~55x45 px | Signal A/B uses raw frame (unaffected). Stereo/YOLO use rectified center zone. |

# 11. Summary & Recommended Next Steps

This pipeline achieves cup volume estimation using exactly 2 cameras mounted on a rigid dual-nozzle block, with a fixed 3 cm stereo baseline and a wide-angle fisheye lens (128-160 degrees). The architecture handles 0, 1, or 2 cups automatically with no hardware changes.

## 11.1 Key Design Decisions

- CAM-L uses the raw (non-rectified) frame for Signal A/B. Fisheye distortion is implicitly absorbed by the linear calibration fit. This preserves the full 80-pixel row-mean precision and the +/-0.12 cm accuracy.

- Both cameras use rectified frames for stereo disparity and YOLO. Fisheye undistortion is mandatory before any geometric calculation involving distances or widths.

- Frame splitting at column 40 enables dual-cup mode for Signal A/B at a minor cost of +/-0.08 cm additional error. The same calibration constants m and c apply to both ROIs since the nozzle block moves as one unit.

- H_cup = H_nozzle - Z_rim is fully independent of whether the user has moved the nozzle toward the cup. The lazy user problem is eliminated.

- The paired-cup constraint (same size when 2 cups present) means both cups can share the same volume output if desired, reducing compute by skipping the second full pipeline.

## 11.2 Recommended Next Steps

- Procure final lens hardware and measure actual FoV to determine whether standard radial model (< 120 deg) or Kannala-Brandt fisheye model (>= 120 deg) is needed.

- Print chessboard calibration target and perform stereo fisheye calibration (Section 3.2) on the physical nozzle block prototype.

- Measure actual effective focal length f on the rectified frame (Section 3.3).

- Perform Signal A/B linear calibration (Section 3.1) with final lens at 4+ known heights.

- Train YOLO model with cup rim samples captured from the actual wide-angle top-down view, including transparent cups and single/dual configurations.

- Implement and test the full integration pipeline (Section 8) on the target edge device.

- Validate volume accuracy against physical measurements across at least 5 cup types.

Critical Verification Before finalizing the lens choice, physically verify that the visible tray area in rows 5-20 of the camera frame is NOT fully occluded by the cup rims when the nozzle is at its lowest position. Signal A/B requires at least a few pixels of bright tray to be visible in the upper portion of each ROI.